# Quickstart Guide – Sim App

[These instructions assume you have NEPI Engine software running on your computer and an Ethernet-enabled edge-compute hardware platform. The simulators run in Docker on your computer; the setup below installs Docker if it is not already installed.]

## Run these commands from your computer

- Use your computer that is connected to a NEPI device for this quickstart guide

## Clone the NEPI simulators repo

- Clone the NEPI simulators repo:

cd ~/

git clone https://github.com/nepi-engine/nepi_simulators.git



## Clone the NEPI drones repo

- Clone the NEPI drones repo:

git clone https://github.com/nepi-engine/nepi_drones.git



## Set up the NEPI simulators

- Run the NEPI simulators setup script; accept the license and enter your password when prompted:

./nepi_simulators/scripts/nepi_simulators_setup.sh

- When asked for the NEPI device address and the NEPI storage share user, press Enter to keep the defaults (192.168.179.103 and nepi).

- If the setup says you were just added to the docker group, log out of your computer and back in, then open a new terminal. Otherwise, run:

sbrc



## Download the simulator image

- Download the NEPI simulator image (numurusinc/nepi_simulations on Docker Hub), which contains Gazebo and MuJoCo. The first download takes a while:

nepi_simulators_pull



## Check connection to your NEPI device

- Check your connection to your NEPI device. Expected output: “NEPI Connected at …”:

pingn



## Start Gazebo

- Start the Gazebo simulator, giving the nepi storage password (nepi); enter your password if prompted. Expected output: “The gazebo simulator is running”:

nepi_simulators_start gazebo nepi

- To use MuJoCo instead, run nepi_simulators_start mujoco nepi. One simulator runs at a time; starting one stops the other.



## Deploy and build the NEPI drones repo

- Deploy the NEPI drones repo to your NEPI device; enter your password when prompted:

./nepi_drones/deploy_nepi_source.sh

- SSH into your NEPI container:

sshn

- Now you are in your NEPI container. Build the changes you deployed; enter the NEPI password when prompted:

nepibld

- Start your RUI in the NEPI container:

nepistart



## Navigate to the sim app in the RUI

- Connect to the RUI in your browser at the following link: http://192.168.179.103:5003/

- Go to SYSTEM > Apps Manager. Select Sim Connector and enable it

## Open the sim app

- Go to AUTOMATION > Sim Connector. This is your sim app page. Below will be a camera view of the simulator that is running on your computer. Below are the relevant settings.

- Simulator: This is your active simulator; keep it selected as the one you started on your computer (Gazebo or MuJoCo).

- Robot Config: This is your robot selection. To update your simulator, use the `Use Open Sim` or `Deploy` buttons below

- Environment Config: This is your environment selection. To update your simulator, use the `Use Open Sim` or `Deploy` buttons below

- Use Open Sim: This button stops the current simulation and starts it again with your selected robot and environment configs

- Kill: This button stops the current simulation

- Deploy: This button appears when you do not have a running simulation. Pressing it will start a new simulation with your selected robot and environment configs

## Open the sim rover device

- Go to DEVICES > ROBOTS and select sim_rover. This is your sim rover device page.

- Under PROCESS CONTROLS, set Select Control Type to Manual or Autonomous to drive the rover.



## Stop the simulator

- When you are done, stop the simulator on your computer:

nepi_simulators_stop



## Tutorials

The Nepi tutorials can be found at https://www.nepi.com/tutorials/